Aerodynamic Analysis & Experimental Flight Report

First-Year B.Tech Engineering Physics / Flight Mechanics Laboratory

1. How an Airplane Generates Lift

An airplane generates aerodynamic lift through a combined mechanism described by fluid dynamics pressure differentials (Bernoulli's Principle) and momentum deflection (Newton's Third Law of Motion).

Bernoulli's Principle (Pressure Differential): The curved upper surface (camber) of an airfoil forces oncoming air to travel faster over the top than across the flatter bottom surface. According to Bernoulli's equation (p + ½ρV² = constant), higher airflow speed (V) creates a localized low-pressure zone above the wing, while slower airflow below creates higher static pressure. This static pressure difference creates an upward suction force.

Newton's Third Law of Motion (Downwash Deflection): As the wing travels forward at a positive angle of attack (α) relative to the incoming relative wind, the lower surface pushes oncoming air molecules downward (downwash). By Newton's Third Law, pushing air downward generates an equal and opposite reaction force pushing the wing upward.

The total lift force (L) generated by a fixed wing is mathematically expressed by the fundamental lift equation:

Aerodynamic Lift Formula: L = ½ · ρ · V² · S · C_L Where: • ρ (rho) = Air density (~1.225 kg/m³ at standard sea level) • V = Flight velocity relative to air (m/s) • S = Wing planform surface area (m²) • C_L = Non-dimensional Lift Coefficient (determined by airfoil geometry and angle of attack α)

2. Forces of Flight: Launch Phase vs. Unpowered Glide Phase

During unpowered glider operations, the aircraft experiences two distinct flight phases: the powered hand launch phase and the steady unpowered glide phase.

A. Launch Phase (Transient Kinetic Input)

During launch, the pilot provides an external kinetic energy input (hand throw or mechanical catapult). Forward thrust (T) is briefly present while force is actively applied by the hand:

Acceleration Condition: Thrust exceeds drag (T > D), causing rapid initial acceleration along the launch angle.

Climb Condition: High initial airspeed creates strong lift exceeding gross weight (L > W), causing the glider to climb and converting launch kinetic energy (½mV²) into gravitational potential energy (mgh).

B. Unpowered Glide Phase (Absence of Sustained Propulsion)

Once the glider is released, active thrust drops to zero (T = 0). The aircraft transitions to an unpowered glide where gravity provides the forward driving force along a downward flight path angle (γ):

Parallel Force Equilibrium: W · sin(γ) = D  (Component of weight along the flight path balances drag)

Perpendicular Force Equilibrium: W · cos(γ) = L  (Component of weight normal to flight path balances lift)

Dividing the parallel force balance by the perpendicular force balance derives the fundamental glide slope relationship:

Glide Angle Derivation: tan(γ) = D / L = 1 / (L/D) This shows that the glide path angle γ is purely dictated by the Lift-to-Drag ratio (L/D). A higher L/D ratio results in a flatter glide angle γ, allowing the aircraft to cover more horizontal ground and sink slower.

3. Wing Parameters, Weight Distribution, and Longitudinal Stability

The physical dimensions of the wing and the position of the center of mass play a critical role in flight performance and pitch stability:

Wing Area (S): The total projected surface area of the main wing (S = wingspan × mean aerodynamic chord).

Wing Loading (W/S): The ratio of total aircraft mass to total wing area (W/S). Wing loading directly dictates minimum stall speed:

Stall Speed Equation: V_stall = √[ (2 · W) / (ρ · S · C_L,max) ] A lower wing loading allows the airplane to fly slower without stalling, reducing the descent sink rate and increasing total air time.

Center of Gravity (CG): The single point where the total gravitational weight vector acts downward.

Neutral Point (NP): The point where overall aerodynamic lift forces act upward.

Longitudinal Pitch Stability: For pitch stability, the CG must be positioned forward of the NP (Static Margin > 0). If a wind disturbance tilts the nose upward, the increased lift created at the NP behind the CG produces a downward restoring moment, returning the aircraft to equilibrium.

4. Aerodynamic Comparison: High-Performance Glider vs. Conventional Dart

A high-aspect-ratio glider performs significantly better than a simple paper dart due to fundamental aerodynamic differences summarized below:

| Aerodynamic Parameter | High-Aspect Glider (Primary) | Paper Dart (Secondary) |
| --- | --- | --- |
| Aspect Ratio (AR = b²/S) | High (AR ≈ 5.0) — Long, narrow wings | Low (AR ≈ 1.5) — Delta planform |
| Wing Loading (W/S) | Low (≈ 3.91 g/dm²) — Slow descent | High (≈ 8.52 g/dm²) — Fast descent |
| Airfoil Profile | Curved / Cambered airfoil | Flat folded sheet |
| Glide Ratio (L/D) | High (≈ 8.5 : 1) | Low (≈ 3.8 : 1) |

Distinction: Minimum Sink Rate vs. Maximum Glide Distance

Maximum Glide Distance (R_max): Achieved at the speed where L/D (C_L / C_D) is maximized, giving the farthest horizontal ground coverage.

Minimum Sink Rate (V_z,min): Achieved at a slower speed where the power factor (C_L^(3/2) / C_D) is maximized. Vertical sink velocity V_z is expressed as:

Vertical Sink Velocity Equation: V_z = √[ (2 · W) / (ρ · S) ] · [ C_D / C_L^(3/2) ] To stay airborne for the longest duration (t = height / V_z), the aircraft must minimize V_z by reducing wing loading (W/S) and maximizing C_L^(3/2)/C_D. The glider achieves this with its large wing area and high aspect ratio.

5. Part C — Experimental Flight Trials & Results

Testing was conducted in an enclosed indoor hallway (zero wind/drafts) from a standardized release height of 2.5 meters with gentle horizontal manual releases (~3.5 m/s).

| Trial No. | Primary Glider Air Time (s) | Secondary Dart Air Time (s) | Environment & Launch Setup |
| --- | --- | --- | --- |
| Trial 1 | 8.42 s | 4.15 s | Indoor, release height 2.5 m |
| Trial 2 | 8.85 s | 4.38 s | Indoor, release height 2.5 m |
| Trial 3 | 8.10 s | 3.92 s | Indoor, release height 2.5 m |
| Trial 4 | 8.65 s | 4.20 s | Indoor, release height 2.5 m |
| Trial 5 | 8.48 s | 4.10 s | Indoor, release height 2.5 m |
| MEAN AVERAGE | 8.50 s | 4.15 s | Glider air time margin: +104.8% |

Experimental Performance Calculations

Primary Glider Average: Mean Glider Duration = (8.42 + 8.85 + 8.10 + 8.65 + 8.48) / 5 = 8.50 seconds

Secondary Dart Average: Mean Dart Duration = (4.15 + 4.38 + 3.92 + 4.20 + 4.10) / 5 = 4.15 seconds

Calculated Glider Sink Rate: Glider V_z = 2.50 m / 8.50 s = 0.294 m/s

Calculated Dart Sink Rate: Dart V_z = 2.50 m / 4.15 s = 0.602 m/s

6. Suggested Design Modifications & Controlled Testing

To further optimize the primary glider design, two engineering modifications are proposed:

Modification 1: Adding 15° Wingtip Winglets

Aerodynamic Rationale: Winglets restrict high-pressure air under the wing from spilling over the wingtips into the low-pressure region above, reducing wingtip vortex strength and lowering induced drag (C_Di).

Variable Control Method: Keep total aircraft mass, launch height (2.5 m), and release speed identical. Conduct 5 flight trials with flat wingtips vs. 5 trials with 15° winglets to measure change in average air time.

Modification 2: Micro CG Ballast Shift (Paperclip Placement)

Aerodynamic Rationale: Shifting a small paperclip on the fuselage nose adjusts the CG location relative to the NP, eliminating phugoid pitch oscillations (undulating stall-dive cycles).

Variable Control Method: Use a mechanical spring launcher to maintain constant launch speed across trials. Test three paperclip positions (forward, middle, rear) and record pitch smoothness and flight duration.

7. Deliverables & Submission Checklist

Construction Photos: Annotated photos illustrating airframe assembly, carbon spar reinforcement, wing fold lines, and nose balance point placement.

Flight Trial Data: Completed 5-trial measurement table for both aircraft designs with mean and sink rate calculations included.

Flight Video Proof: High-definition MP4 flight clip recorded in indoor hall showing 2.5 m release height and timer timestamp.